HOP 0507 - Onset of CMEs - coordination with Proba-3/ASPIICS coronagraph (2026/09/03-2026/09/04) (2026/09/06-2026/09/07)(2026/09/09-2026/09/10)

Filters: Be-thin (T > 3MK), Be-med (hot plasma emission)

FOV: 512" x 512"

Target: selected AR and the near-limb region across which the CME will likely travel.

Cadence: 30 sec (or higher, depending on telemetry) with automatic exposure control

XOB:  Similar to #1D4A (120s), but at a higher cadence

HOP 0516 - EIS support for ground based observations of total eclipse of 12 August 2026 (2026/08/12-2026/08/12)

-NO XRT Request

HOP 0521 - [SOOP: Major Flare] High Cadence Solar Flare Obs (2026/08/18-2026/08/20) (2026/09/18-2026/09/18)

HOP 476/489 with AEC 3.

XOB:  #1D0D (60s cad),  #1D2D (90s cad)

Flare XOB:  #1D27 (2s) or #1D26 (6s)

HOP 0528 - Dynamics of the bright points in the corona and in the low atmosphere (2026/09/17-2026/09/19)

Request: HOP 373 (like 2019 Oct 01 with THEMIS): Standard 2-filter active region program at a 1-2 minute cadence

HOP 0534 - Flare Abundance Evolution and Low Atmospheric Dynamics from Coordinated EIS, IRIS, and GST/FISS Observations (2026/08/18-2026/08/23)

XRT optional.

384x384 FOV with two filters ( Al-poly and thin-Be)

AR obs. Program

HOP 0535 - Coordination with GST to study the center-to-limb variation of MHD waves in coronal holes and the faculae. (2026/08/14-2026/08/17) (2026/08/14-2026/08/17)

- No XRT Request

HOP 0537 - Characteristics of Magnetic Reconnection in Solar Flares from Coordinated Hinode, IRIS, and GST/VIS Sit-and-Stare Observations (2026/08/24-2026/08/27)

- Optional XRT

- 384x384 FOV with two filters ( Al-poly and thin-Be)

HOP 0540 - [SOOP: FIlaments] Thermal-Radiative condensation evolution and equilibrium (2026/08/12-2026/08/13) (2026/08/26-2026/08/27)(2026/08/30-2026/08/30)

No request XRT

HOP 0541 - [SOOP: Fast Wind] Solar wind origin regions in coronal holes (2026/08/17-2026/08/17) (2026/08/21-2026/08/21) (2026/08/23-2026/08/23) (2026/08/26-2026/08/26)

For on-disk coronal hole, run the following XRT study.

Filters :

Al-poly, Be-thin

Field of view: 512x512

XOB: CME Watch programs

#1B96, #1D23, #1C51 (30sec) , #1C36 (30sec)

Previous HOP 522 and 502

HOP 0542 - [SOOP AR-Long-Term] - stereo spectropolarimetry with SolO/PHI and SST/CRiSP (2026/08/17-2026/08/26) (2026/08/17-2026/08/26)

Request: If the telemetry allows it. Hopefully we will catch a flaring region so "flare mode" would be preferable.

HOP 0543 - [SOOP:] ASPIICS coordination and limb mosaic (2026/08/29-2026/08/30)

Request: Large FoV, low cadence observations as telemetry allows

Filters: Al-mesh or Al-poly

XOB: Possibly a Single filter Al-poly CME Watch program. Target(s) of interest: Limb.

HOP 0544 - [SOOP: COORD_CALIBRATION] Coordinated calibration with Solar Orbiter SPICE and PHI (2026/08/28-2026/08/29)

No XRT Request. Possibly CME Watch program. The target will consists of 4 pointings from the limb (West preferred) to the Sun centre.

HOP 0545 - [SOOP: R_SMALL_HRES_MCAD_Polar-Observations] High Latitude Polar Survey (2026/09/16-2026/09/18)

No XRT Request. Possibly CME Watch program. The target will be the North Pole.

HOP 0546 - [SOOP: R_SMALL_HRES_LCAD_Composition-vs-Height] Tracking the composition through the solar atmosphere layers (2026/09/19-2026/09/19)

No XRT Request. Possibly CME Watch program. Target: Coronal hole or active region boundary on disk.